EE 8205 – EMBEDDED COMPUTER SYSTEMS

TUTORIAL LAB REPORT

uVision for ARM Cortex M3/M4 CPUs

PREPARED BY: MUHAMMAD IMADULLAH FAROOQUI

SUBMITTED TO: DR. GUL N. KHAN

TASK1 – EXPLORING THE DEBUG SESSION WINDOWS RUNNING Blinky.c

Observing ADC values on logic analyzer after changing from A/D Converter Window

Changing the ADC VALUE from 0V to 3.3V to 1.2V

TASK 2 – PRINTING ADC VALUE AND OBSERVING TOGGLING LEDs

Printing current ADC values while changing from A/D Converter window

GPIOs connected to LEDs toggling from the IRQ handler

TASK 3 (BONUS)  – READING & PRINTING JOYSTICK POSITION (Blinky.c modified)

Printing Joystick Position ‘RIGHT’ & ‘LEFT’ while toggling input from Task Interface ‘Pins’

Toggling Joystick Input pins 20,23,24,25 & 26

TASK 3 (BONUS) - MODIFIED CODE HIGHLIGHTED IN GREEN - Blinky.c

| #include <stdio.h> #include "Blinky.h" #include "LPC17xx.h" // Device header #include "Board_LED.h" // ::Board Support:LED #include "Board_ADC.h" // ::Board Support:A/D Converter #include "Board_Joystick.h" static char text[10]; static char text_js[20]; // variable to trace in LogicAnalyzer static volatile uint16_t AD_dbg; static volatile uint16_t joystick_dbg; uint16_t ADC_last; // Last converted value int main (void) { int32_t res; int32_t joystick_res; uint32_t AD_sum = 0U; uint32_t AD_cnt = 0U; uint32_t AD_value = 0U; uint32_t AD_print = 0U; LED_Initialize(); // LED Initialization ADC_Initialize(); // ADC Initialization Joystick_Initialize(); //Inititalize the Joystick SystemCoreClockUpdate(); SysTick_Config(SystemCoreClock/100U); // Generate interrupt each 10 ms if (AD_value != AD_print) { sprintf(text, "0x%04X", AD_value); // format text } | while (1) { //Loop forever res = ADC_GetValue(); // AD converter input joystick_res = Joystick_GetState(); //READ JSTIC VAL if (res != -1) { // If conversion has finished ADC_last = (uint16_t)res; AD_sum += ADC_last; // Add AD value to sum if (++AD_cnt == 16U) { //average over 16 values AD_cnt = 0U; AD_value = AD_sum >> 4; // average /by 16 AD_sum = 0U; }} printf("AD value: %s\r\n", text); if(joystick_res == 0) {printf("At Rest\r\n");} if(joystick_res == 1) { printf( "Position SELECT\r\n" );} if(joystick_res == 2) {printf("Position UP\r\n");} if(joystick_res == 4 {printf("Position DOWN\r\n");} if(joystick_res == 8) {printf("Position LEFT\r\n");} if(joystick_res == 16){ printf("Position RIGHT\r\n");} AD_print = AD_value; // Get unscaled value for AD_dbg = (uint16_t)AD_value; joystick_dbg = (uint16_t)joystick_res; if (clock_1s) { clock_1s = 0; |
| --- | --- |